# Installation instructions

Turn the power off

Switch off the circuit at the breaker before you start. Installation must be done by a qualified electrician.

# In the box

- SP-24 panel

- Driver box with 1.5 m lead

- 4 safety cables

- This guide

# Tools you need

Wire strippers, a 3 mm flat screwdriver, a ladder and a voltage tester.

# Steps

- Remove a ceiling tile and check the opening is 600 x 1200 mm.

- Fix the safety cables to the structure above.

  - Use the supplied anchors in concrete.

  - Keep the cables slack by about 50 mm.

- Connect the supply to the driver box: L (brown), N (blue), earth (green/yellow).

- Lift the panel into the grid and clip the safety cables to the frame.

- Restore power and test all modes.

Figure 1: Clipping the safety cable to the frame

# Wiring

| Terminal | Wire colour | Connect to |
| --- | --- | --- |
| L | Brown | Live |
| N | Blue | Neutral |
| PE | Green/yellow | Earth |
| DA+ / DA- | Grey / white | DALI bus (polarity free) |

Leave 300 mm of slack in the supply so the panel can be lowered for service.

# Troubleshooting

| Problem | What to check |
| --- | --- |
| Panel stays off | Breaker, supply wiring and the driver LED |
| Flicker when dimmed | Dimmer compatibility; use 0-10 V or DALI |